Overview of the input and output variables of the irrigation model

| Variable Name | Format and Units | Description | Source |
| --- | --- | --- | --- |
| INPUT DATA |  |  |  |
| Irr_method | Text, menu (surface, drip, sprinkler, subterranean) | Irrigation method selection | USER |
| n_emitter | Integer (-) | Number of sprinklers, applicable if irrigation method is "sprinkler". | USER |
| s_rate | Decimal (l/h) | Sprinkler flow rate. | USER |
| n_drip_per_line | Integer (-) | Number of dripping lines per row of crop (1 or 2), applicable for "drip" and "subterranean" methods. | USER |
| d_spacing | Decimal (m) | Spacing between drippers, applicable for "drip" and "subterranean" methods. | USER |
| d_rate | Decimal (l/h) | Dripper flow rate, applicable for "drip" and "subterranean" methods. | USER |
| w_length | Decimal (m) | Wetted length, applicable for "drip" and "subterranean" methods. | USER |
| d_depth | Decimal (m) | Depth of dripper, applicable for "subterranean" method. | USER |
| Season_start | Date (DD/MM/YYYY) | Planting or budbreak day. | USER |
| Season_end | Date (DD/MM/YYYY) | Harvest date. | USER |
| Climate zone | Integer (1 or 2) | Climate classification where 1 is Mediterranean and 2 is Arid. | PLATFORM |
| crop | Text, menu | Crop selection from predefined list. | USER |
| crop_table | Dataframe (54 x 23) Units indicated in the crop_table_description.docx file | Dataframe containing the crop and irrigation practices’ coefficients specific for each crop (among the list of crops available) and climate zone (arid, semi-arid) |  |
| Area_crop | Decimal (m²) | Crop area in square meters. | USER |
| plant_spacing | Decimal (m) | Spacing between each plant in a row | USER |
| line_spacing | Decimal (m) | Spacing between each row | USER |
| P_sand | Decimal (%) | Percentage of sand in soil composition. | USER |
| P_clay | Decimal (%) | Percentage of clay in soil composition. | USER |
| P_silt | Decimal (%) | Percentage of silt in soil composition. | USER |
| Soil_SAT_user | Decimal (%) | Water content at saturation indicated by the user | USER |
| theta_fc_user | Decimal (%) | Soil moisture content at field capacity indicated by the user | USER |
| theta_wp_user | Decimal (%) | Soil moisture content at wilting point indicated by the user | USER |
| Ks | Decimal (cm/h) | Saturated hydraulic conductivity of soil. | USER |
| Pb | Decimal (g/cm³) | Soil bulk density. | USER |
| P_om | Decimal (%) | Percentage of organic matter content | USER |
| lat | Decimal (Degrees) | Latitude of farm location. | PLATFORM |
| elev | Decimal (m) | Elevation of the farm location. | PLATFORM |
| temp_past | Array (168 elements, °C) | Air temperature data from past 168 hourly steps. | SENSOR |
| precipitation_past | Array (168 elements, mm) | Precipitation data from past 168 hourly steps. | SENSOR |
| sw_rad_past | Array (168 elements, W/m²) | Short-wave solar radiation from past 168 hourly steps. | SENSOR |
| RH_past | Array (168 elements, %) | Relative humidity from past 168 hourly steps. | SENSOR |
| wind_past | Array (168 elements, m/s) | Wind speed from past 168 hourly steps. | SENSOR |
| date_past | Array (168 elements, DD/MM/YYYY) | Dates of past 168 hourly steps. | SENSOR |
| hh_past | Array (168 elements, hh) | Hours of past 168 hourly steps. | SENSOR |
| temp_fut | Array (168 elements, °C) | Forecasted air temperature data for next 168 hourly steps. | SENSOR |
| precipitation_fut | Array (168 elements, mm) | Forecasted precipitation data for next 168 hourly steps. | SENSOR |
| sw_rad_fut | Array (168 elements, W/m²) | Forecasted short-wave solar radiation for next 168 hourly steps. | SENSOR |
| RH_fut | Array (168 elements, %) | Forecasted relative humidity for next 168 hourly steps. | SENSOR |
| wind_fut | Array (168 elements, m/s) | Forecasted wind speed for next 168 hourly steps. | SENSOR |
| date_fut | Array (168 elements, DD/MM/YYYY) | Dates of future 168 hourly steps. | SENSOR |
| hh_fut | Array (168 elements, hh) | Hours of future 168 hourly steps. | SENSOR |
| irr_given_vol_original | Array (168 elements, m3/h) | Past irrigation flow rate data for 168 hourly steps. | SENSOR |
| moisture_sensor_raw | matrix (168 x X, X = number of sensors, %) | Soil moisture readings from sensors over a column of soil, e.g., at 10, 20, and 30 cm depths (if X = 3). | SENSOR |
| Sensor_spacing | Decimal (m) | Distance between each depth recording by a sensor. e.g., if sensor is measuring at 10, 20, and 30 cm depth, then sensor_spacing is 0.10 m | PLATFORM |
| extremes_previous_run | Array (2 elements, - ) | Maximum and minimum soil moisture values recorded in previous runs of the model. | PLATFORM |
| OUTPUT DATA |  |  |  |
| Daily_varaibles_final | Dataframe (15x4, YYYY-MM-DD, m3, m3, %) | Dataframe containing the date (YYYY-MM-DD), suggested and deficit irrigation volume (m3) and the soil moisture (%) aggregated at daily scale for the two-weeks simulation (15 days) |  |
| hourly_variables_final | Dataframe (337x4, DD-MM-YYYY, m3, m3, %) | Dataframe containing the date (YYYY-MM-DD), suggested and deficit irrigation volume (m3) and the soil moisture (%) at hourly scale for the two-weeks simulation (337 hours) |  |
| Irrigation_dates | Dataframe (1x4 DD-MM-YYYY) | Dataframe containing the dates of the day when the simulation occurs (Today) and the dates of Early (no water stress), Late (moderate water stress) and Limit (high water stress) irrigation suggestions |  |
| SM_extremes_previous_run | Array (2x1, %) | Dataframe containing the maximum and minimum values recorded by the soil moisture sensor over the history of the past simulations (e.g., at every run, the model identifies the max and min values in the dataset and compares it with the max and min of the past simulations, then it takes the absolute max and min). This dataframe is saved in the platform and updated at each run |  |
| kc_correction | Decimal (-) | Correction of crop coefficient (kc), with values ranging from 0.5 to 1.5 (1 as default). At each iteration, the kc is modulated by the kc_correction coefficient to compensate the past irrigation events. This integer is saved in the platform and updated at each run |  |