| Variable Name | Units and Format | Description |
| --- | --- | --- |
| Data from users |  |  |
| irr_method | Text, menu (drip, sprinkler, subterranean) | Irrigation method selection. |
| n_emitter | Integer | Number of sprinklers, applicable if irrigation method is "sprinkler". |
| s_rate | l/h | Sprinkler flow rate. |
| n_drip_per_line | Integer | Number of dripping lines per row of crop (1 or 2), applicable for "drip" and "subterranean" methods. |
| d_spacing | m | Spacing between drippers, applicable for "drip" and "subterranean" methods. |
| d_rate | l/h | Dripper flow rate, applicable for "drip" and "subterranean" methods. |
| w_length | m | Wetted length, applicable for "drip" and "subterranean" methods. |
| d_depth | m | Depth of dripper, applicable for "subterranean" method. |
| Season_start | Date (dd/mm/yyyy) | Planting or budbreak day. |
| Season_end | Date (dd/mm/yyyy) | Harvest date. |
| Selezione crop | Text, menu | Crop selection from predefined list. |
| Area_crop | m² | Crop area in square meters. |
| Sesto d’impianto | m x m (optional) | Plant spacing in the row and between rows. |
| P_sand | % | Percentage of sand in soil composition. |
| P_clay | % | Percentage of clay in soil composition. |
| P_silt | % | Percentage of silt in soil composition. |
| Soil_AWC | % | Available water content in soil at saturation. |
| Soil_AWC_sensor | % | Water content at saturation recorded by a sensor. |
| theta_fc | % | Soil moisture content at field capacity. |
| theta_wp | % | Soil moisture content at wilting point. |
| Ks | cm/h | Saturated hydraulic conductivity of soil. |
| Pb | g/cm³ | Soil bulk density. |
| Data from platform |  |  |
| Climate zone | Integer (1 or 2) | Climate classification where 1 is Mediterranean and 2 is Arid. |
| lat | Degrees | Latitude of farm location. |
| elev | m | Elevation of the farm location. |
| temp_past | °C, array (96 elements) | Air temperature data from past 96 hourly steps. |
| precipitation_past | mm, array (96 elements) | Precipitation data from past 96 hourly steps. |
| sw_rad_past | W/m², array (96 elements) | Short-wave solar radiation from past 96 hourly steps. |
| RH_past | %, array (96 elements) | Relative humidity from past 96 hourly steps. |
| wind_past | m/s, array (96 elements) | Wind speed from past 96 hourly steps. |
| date_past | Date (dd/mm/yyyy), array (96 elements) | Dates of past 96 hourly steps. |
| hh_past | Hour (hh), array (96 elements) | Hours of past 96 hourly steps. |
| temp_fut | °C, array (96 elements) | Forecasted air temperature data for next 96 hourly steps. |
| precipitation_fut | mm, array (96 elements) | Forecasted precipitation data for next 96 hourly steps. |
| sw_rad_fut | W/m², array (96 elements) | Forecasted short-wave solar radiation for next 96 hourly steps. |
| RH_fut | %, array (96 elements) | Forecasted relative humidity for next 96 hourly steps. |
| wind_fut | m/s, array (96 elements) | Forecasted wind speed for next 96 hourly steps. |
| date_fut | Date (dd/mm/yyyy), array (96 elements) | Dates of future 96 hourly steps. |
| hh_fut | Hour (hh), array (96 elements) | Hours of future 96 hourly steps. |
| irr_given_vol | m³/h, array (96 elements) | Past irrigation flow rate data for 96 hourly steps. |
| moisture_sensor_raw | %, matrix (96 x X, X = number of sensors) | Soil moisture readings from sensors over a column of soil, e.g., at 10, 20, and 30 cm depths (if X = 3). |
| Sensor_spacing | m | Distance between each depth recording by a sensor. e.g., if sensor is measuring at 10, 20, and 30 cm depth, then sensor_spacing is 0.10 m |
| max_previous_run | % | Maximum soil moisture value recorded in previous model run. |
| OUTPUT DATA |  |  |
| Irr_vol | m3, array (192 elements) | Daily irrigation volume required according to soil water balance model |
| Irr_suggested | m3, array (192 elements) | Suggested irrigation volume provided in order to keep water stress coefficient above 0.8 |
| Irr_limit | m3, array (192 elements) | Irrigation volume provided in order to keep water stress coefficient above 0.5 |
| Irrigation_event_suggested | Table (X rows, 3 columns, where X= number of irrigation events during future data) | Summary table resuming the number of irrigation events. In the first column the day of irrigation is reported, in the second the volume of water provided during each irrigation, in the third the hours of irrigation in order to provide that amount of irrigation |
| Irrigation_event_limit | Table (X rows, 3 columns, where X= number of irrigation events during future data) | Summary table resuming the number of irrigation events. In the first column the day of irrigation is reported, in the second the volume of water provided during each irrigation, in the third the hours of irrigation in order to provide that amount of irrigation |